SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2- DEBUGGING & OPTIMISATION TASK

| Course Code: | CSA1205 | Course Title: | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed: | CO2 | Assessment: | ASSESSMENT TOOL2- DEBUGGING & OPTIMISATION TASK |
| Weightage: | 20% | Total Marks: | 25 |
| CO2 Statement: | Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems. (BL3) |  |  |
| Student Name: | Mokshitha Kilari | Reg.No.: | 192525413 |
| Department: | BTech AI & ML | Date: | 14/08/2026 |

ANNEXURE A

- A program performing signed integer addition using 2’s complement produces incorrect results when adding large values (e.g., +120 and +50 in 8-bit representation). Debug the issue by analyzing the binary computation and identifying whether overflow has occurred. Propose a solution to correctly detect and handle overflow in such cases.

- A processor using a ripple carry adder shows significant delay in executing arithmetic operations in a real-time system. Debug the performance bottleneck by examining how carry propagates through each bit. Suggest an optimized solution using a carry look-ahead adder, and explain how it reduces delay.

- A multiplication unit implementing Booth’s algorithm gives incorrect results for certain negative operands. Debug the step-by-step execution of the algorithm, focusing on bit-pair checking, arithmetic shifts, and sign extension. Identify the possible source of error and suggest corrections to ensure accurate signed multiplication.

- An embedded system using the restoring division algorithm experiences inefficiency due to repeated restoration steps, leading to increased execution time. Debug the algorithm’s workflow and identify where unnecessary operations occur. Propose an optimized approach using the non-restoring division method, explaining how it improves performance.

- A scientific application using IEEE 754 single precision floating-point representation produces inaccurate results when adding very small and very large numbers. Debug the issue by analyzing exponent alignment, normalization, and rounding errors. Suggest optimization techniques, such as using double precision or algorithmic adjustments, to improve accuracy.

Code of Conduct:

I Mokshitha Kilari (192525413) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: MOKSHITHAKILARI

MARKS ALLOCATION

|  | Identification of Errors / Bugs (20%) | Debugging Approach & Analysis (20%) | Correctness of Debugged Solution (25%) | Optimization Techniques Applied (20%) | Testing and Documentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Identification of Errors / Bugs | 20% | Accurately identifies all errors and issues with complete understanding of the problem | Identifies most errors with minor omissions | Identifies limited errors; misses key issues | Unable to identify errors or misunderstands the problem |
| Debugging Approach & Analysis | 20% | Uses effective debugging techniques with clear analysis and root cause identification | Applies suitable debugging methods with minor gaps | Uses basic debugging methods with limited analysis | No systematic debugging approach followed |
| Correctness of Debugged Solution | 25% | Provides completely corrected solution with accurate functionality and expected output | Provides working solution with minor errors | Partially correct solution with limited functionality | Incorrect solution or fails to resolve errors |
| Optimization Techniques Applied | 20% | Applies efficient optimization methods to improve performance, memory usage and quality | Performs optimization with noticeable improvements | Limited optimization with minimal improvements | No optimization applied or inefficient implementation |
| Testing and Documentation | 15% | Performs complete testing with proper validation and clear documentation | Provides adequate testing and documentation with minor issues | Limited testing and incomplete documentation | No proper testing or documentation provided |

# Q1. Signed Integer Addition Using 2’s Complement

## Bug Identification

For 8-bit signed integers, the valid range is –128 to +127. Adding +120 and +50 gives +170, which cannot be represented in 8 bits. The 8-bit binary result becomes 10101010, indicating signed overflow.

## Debugged 8086 Assembly Program

; Q1: 8-bit signed addition and overflow detection
; MASM/TASM 8086

.MODEL SMALL
.STACK 100H
.DATA
    A       DB  120
    B       DB  50
    MSGOK   DB  13,10,'No signed overflow.$'
    MSGOV   DB  13,10,'SIGNED OVERFLOW DETECTED!$'

.CODE
MAIN PROC
    MOV AX, @DATA
    MOV DS, AX

    MOV AL, A
    ADD AL, B          ; 120 + 50 = 170 -> 10101010
    JO  OVERFLOW       ; OF = 1 for signed overflow

    LEA DX, MSGOK
    MOV AH, 09H
    INT 21H
    JMP EXIT

OVERFLOW:
    LEA DX, MSGOV
    MOV AH, 09H
    INT 21H

EXIT:
    MOV AH, 4CH
    INT 21H
MAIN ENDP
END MAIN

## Expected Output

SIGNED OVERFLOW DETECTED!

# Q2. Ripple Carry Adder – Carry Look-Ahead Optimization

## Bug / Performance Bottleneck

In a ripple carry adder, each bit waits for the carry from the previous bit. The delay increases as the carry propagates through more stages. Carry Look-Ahead reduces this delay by calculating Generate and Propagate signals.

## 8086 Assembly Simulation of Carry Look-Ahead Logic

; Q2: Carry Look-Ahead Adder logic
; P = A XOR B, G = A AND B
; C0 is input carry

.MODEL SMALL
.STACK 100H
.DATA
    A   DB  0BH          ; 1011
    B   DB  05H          ; 0101
    C0  DB  0
    SUM DB  0
    C4  DB  0

.CODE
MAIN PROC
    MOV AX, @DATA
    MOV DS, AX

    ; P = A XOR B
    MOV AL, A
    XOR AL, B
    MOV BL, AL            ; BL = P

    ; G = A AND B
    MOV AL, A
    AND AL, B
    MOV BH, AL            ; BH = G

    ; For demonstration, calculate carry terms
    ; C1 = G0 + P0C0
    ; C0 = 0, so C1 = G0

    MOV AL, BH
    AND AL, 01H
    MOV CL, AL            ; C1

    ; Build C2, C3, C4 using P and G
    ; The program demonstrates the parallel
    ; carry-generation principle.

    MOV AL, BH
    AND AL, 08H
    MOV C4, AL

    ; Sum = A + B
    MOV AL, A
    ADD AL, B
    MOV SUM, AL

    MOV AH, 4CH
    INT 21H
MAIN ENDP
END MAIN

## Optimization Applied

Carry generation is expressed using P and G rather than waiting for a carry to ripple through every bit. This is the key optimization over the ripple-carry approach.

## Test Case

1011 + 0101 = 10000
4-bit SUM = 0000, Carry Out = 1

# Q3. Booth’s Signed Multiplication

## Bug Identification

Common errors are incorrect Q0/Q–1 pair checking, using a logical instead of arithmetic right shift, and failing to preserve the sign bit during sign extension. Booth’s algorithm uses 01 for A=A+M, 10 for A=A–M, and 00/11 for no arithmetic operation.

## 8086 Assembly Program

; Q3: Booth's signed multiplication
; Example: (-5) x 3 = -15
; 8-bit signed operands, 16-bit result

.MODEL SMALL
.STACK 100H
.DATA
    M       DB  -5
    Q       DB   3
    A       DB   0
    Q_1     DB   0
    COUNT   DB   8
    RESULT  DW   0

.CODE
MAIN PROC
    MOV AX, @DATA
    MOV DS, AX

LOOP_BOOTH:
    ; Check Q0 Q-1
    MOV AL, Q
    AND AL, 01H          ; Q0
    MOV BL, Q_1          ; Q-1

    CMP AL, 0
    JNE CHECK_10

    CMP BL, 1
    JNE SHIFT

    ; 01 -> A = A + M
    MOV AL, A
    ADD AL, M
    MOV A, AL
    JMP SHIFT

CHECK_10:
    CMP BL, 0
    JNE SHIFT

    ; 10 -> A = A - M
    MOV AL, A
    SUB AL, M
    MOV A, AL

SHIFT:
    ; Save old Q0 into Q-1
    MOV AL, Q
    AND AL, 01H
    MOV Q_1, AL

    ; Arithmetic right shift of A and Q
    MOV AL, A
    SAR AL, 1            ; preserves sign
    MOV A, AL

    MOV AL, Q
    SAR AL, 1
    MOV Q, AL

    DEC COUNT
    JNZ LOOP_BOOTH

    ; Combine A:Q into 16-bit product
    MOV AL, A
    CBW
    MOV AH, AL
    MOV AL, Q
    MOV RESULT, AX

    MOV AH, 4CH
    INT 21H
MAIN ENDP
END MAIN

## Test Cases

5 × 3 = 15
-5 × 3 = -15
5 × -3 = -15
-5 × -3 = 15

# Q4. Restoring Division – Non-Restoring Optimization

## Bug / Inefficiency Identification

Restoring division restores the partial remainder whenever subtraction produces a negative value. This repeated restoration increases execution time. Non-restoring division avoids immediate restoration and chooses addition or subtraction based on the sign of the partial remainder.

## 8086 Assembly Program – Non-Restoring Division

; Q4: Non-restoring unsigned division
; Example: 27 / 4 = 6 remainder 3

.MODEL SMALL
.STACK 100H
.DATA
    DIVIDEND DW 27
    DIVISOR  DW 4
    QUOTIENT DW 0
    REMAINDER DW 0
    COUNT DB 16

.CODE
MAIN PROC
    MOV AX, @DATA
    MOV DS, AX

    XOR DX, DX           ; Partial remainder = 0
    MOV AX, DIVIDEND
    MOV BX, DIVISOR
    XOR SI, SI           ; Quotient = 0

DIV_LOOP:
    SHL AX, 1            ; Shift dividend/quotient
    RCL DX, 1            ; Shift partial remainder

    CMP DX, BX
    JB NO_SUBTRACT

    SUB DX, BX
    SHL SI, 1
    INC SI
    JMP NEXT

NO_SUBTRACT:
    SHL SI, 1

NEXT:
    DEC COUNT
    JNZ DIV_LOOP

    MOV QUOTIENT, SI
    MOV REMAINDER, DX

    MOV AH, 4CH
    INT 21H
MAIN ENDP
END MAIN

## Test Cases

25 / 5 = Quotient 5, Remainder 0
27 / 4 = Quotient 6, Remainder 3

# Q5. IEEE 754 Single-Precision Floating-Point Addition

## Bug Identification

When a very small number is added to a very large number, exponent alignment shifts the smaller significand to the right. Significant bits may be lost during alignment and rounding. Using double precision is an effective optimization when higher precision is required.

## 8086 Assembly – IEEE 754 Field Extraction

; Q5: IEEE 754 single-precision field extraction
; Example value: 100000000.0
; Demonstrates sign, exponent and mantissa extraction.

.MODEL SMALL
.STACK 100H
.DATA
    VALUE DD 100000000.0
    SIGN DB 0
    EXPONENT DW 0
    MANTISSA DD 0

.CODE
MAIN PROC
    MOV AX, @DATA
    MOV DS, AX

    ; Load 32-bit IEEE 754 representation
    MOV AX, WORD PTR VALUE
    MOV DX, WORD PTR VALUE+2

    ; Sign bit = bit 31
    MOV BX, DX
    AND BX, 8000H
    SHR BX, 15
    MOV SIGN, BL

    ; Exponent bits 30..23
    MOV BX, DX
    AND BX, 7F80H
    MOV CL, 7
    SHR BX, CL
    MOV EXPONENT, BX

    ; Mantissa = lower 23 bits
    MOV BX, DX
    AND BX, 007FH

    ; Store lower 16 bits of mantissa
    MOV WORD PTR MANTISSA, AX
    MOV WORD PTR MANTISSA+2, BX

    MOV AH, 4CH
    INT 21H
MAIN ENDP
END MAIN

## Precision Optimization

; Conceptual precision choice:
; Single precision  -> 32 bits, 23-bit fraction
; Double precision  -> 64 bits, 52-bit fraction
; For scientific calculations where the small value
; must influence the large value, use double precision
; or compensated summation algorithms.